DRAFT NBS Specification

Troldtekt® design solution panels “Tiles T24” installed in exposed T24 profiles in removable, suspended ceiling system

(Suspended Ceiling System)

K40

135	SUSPENDED CEILING SYSTEM

- Please refer to Architect’s drawings ……………………. for locations

- Structural soffit: ………………………

- Ceiling system manufacturer/supplier: Troldtekt A/S, Sletvej 2A, 8310 Tranbjerg J, Denmark Tel: +45 8747 8100 Fax +45 8747 8111 Email: sales@troldtekt.co.uk

- Product reference: Troldtekt® Tiles T24

- Panel size: 600X600mm, 600X1200mm

- Thickness: 25mm

- Panel finish: Natural wood, White 101, Black 207

- Structure: Ultrafine 1mm

- Edge design: tiles 5 T24 / tiles 8 T24

- Ceiling type: Fine fibre wood wool panels meeting EN 13168 and EN 13964 standards. The panels must be available as PEFC (PEFC/09-31-030) - and FSC® (FSC® C115450)-certified.

- Certified “Gold” Cradle to Cradle” and the "Allergy Friendly Product Award" - "Allergy UK".

- Grid form: T24 Troldtekt profiles

- Grid exposure: Exposed

- Suspension system: Troldtekt T24 profiles.

- T24 grid centres: Maximum 600mm

- Hanger centres: Maximum 1200mm

- Ceiling module: 600X600mm, 600X1200mm

- Access: All panel units are installed in suspended, exposed T24 profiles and demountable.

- Ceiling suspension depth: ………..(Variable, min. 200mm from structural soffit)

- Manufacturer’s Troldtekt suspension components to include Main tee profiles, Cross tee profiles, Adjustable hangers and Perimeter angle profiles.

- Insulation:	30 or 50mm stone wool (45kg/m3) acoustic pads (If required to complete the required Absorber Class ceiling system)

325 INSTALLING ADJUSTABLE SUSPENSION HANGERS

- Installation: Install Troldtekt Adjustable hangers vertically and securely the structural soffit. Fasteners to be sufficiently long to provide adequate fix.

- Spacing: As T24 MF Troldtekt grid manufacturer’s requirements.

335 PERIMETER CHANNEL TRIM

- Jointing: Neat and accurate, without lipping or twisting

- External and internal corners: Mitre joints

Intermediate butt joints: Minimize. Use longest available lengths of trim. Align adjacent lengths.

- Fixing: Fix firmly to perimeter wall or other building structure with appropriate fixings.

375 INSTALLING PANEL CEILING SYSTEM

- Installation should be in accordance with manufacturer’s instructions, see www.troldtekt.co.uk  for download.

- Moisture content of boards at time of fixing: Appropriate to end use.

- It is important that the Troldtekt panels are allowed to acclimatize prior to installation. Acclimatization means that the panels must adapt to the temperature and humidity in the room where they are to be installed.

- Panels must be installed in stack bond / grid.

- Cutting: Cut panels neatly and accurately in accordance with manufacturer’s instructions.

- Fixing: Lay Troldtekt panels in the grid. When installing 600 × 600mm panels, it is important to ensure that the wood wool fibres run mainly parallel in one direction..

- Place 30 or 50mm stone wool (45kg/m3) acoustic pads over Troldtekt panels and fit tightly between the T24 grid sections, cutting when necessary.

- Movement joints: To coincide with any movement joints in the surrounding structure.

- Exposed board edges are to be site painted with Troldtekt Touch-up Paint.

395 INTEGRATED SERVICES

- General: Position services accurately, support adequately. Align and level in relation to the ceiling and suspension system. Do not diminish performance of ceiling system.

- Small fittings: Support with rigid backing boards or other suitable means. Do not damage or distort the ceiling.

- Surface spread of flame rating of additional supporting material: Not less than ceiling material.

- Services outlets:

- Supported by ceiling system: Provide additional hangers.

- Independently supported: Provide flanges to support or trim ceiling system.